LiveGuard-EHMS

Immediate Development Guide: Laptop + Arduino Sensor Prototype
(No Raspberry Pi Required)

# 1. Overview: How Your Laptop Acts as the Edge Device

While awaiting your Raspberry Pi, your Laptop functions as the complete Edge Intelligence Node. The Arduino acts strictly as a high-speed analog acquisition bridge, capturing real-time ECG signals at 360 Hz and streaming them over USB Serial into the Python pipeline running on your PC.

The Laptop executes:
• Digital Signal Filtering (0.5–40 Hz Butterworth Bandpass + 50 Hz Notch filter)
• Real-Time Pan-Tompkins QRS / R-Peak Detection
• 180-Sample Beat Normalization & Windowing
• PyTorch Stage 1 CNN AI Inference (< 3ms per heartbeat)
• WebSocket Live Telemetry Server (for browser dashboard display)

# 2. Step 1: Wire the AD8232 to Your Arduino

Use jumper wires to connect the AD8232 ECG sensor to your Arduino Uno or Nano as follows:

| AD8232 Sensor Pin | Arduino Pin | Wire Color | Description |
| --- | --- | --- | --- |
| 3.3V | 3.3V | Red | Power supply (Do NOT connect to 5V) |
| GND | GND | Black | Ground reference |
| OUTPUT | Pin A0 | Yellow | Analog ECG voltage signal (0-1023) |
| LO+ | Digital Pin 2 | Blue | Leads-Off Detection Positive |
| LO- | Digital Pin 3 | Green | Leads-Off Detection Negative |
| SDN | Unconnected | — | Leave open / unconnected |

# 3. Step 2: Stick the 3 Electrodes on Your Body

Connect the 3.5mm electrode cable into the AD8232 jack and attach 3 disposable gel pads:
1. RED (RA - Right Arm): Stick below your right collarbone (clavicle) or on right inner wrist.
2. YELLOW (LA - Left Arm): Stick below your left collarbone (clavicle) or on left inner wrist.
3. GREEN (RL - Right Leg): Stick on your lower right abdomen / ribcage (Ground reference electrode).

Important: Clean skin with an alcohol wipe to remove natural oils for clear waveforms with no drift.

# 4. Step 3: Upload Firmware to Arduino

1. Open the Arduino IDE on your laptop.
2. Open file: C:\LiveGuard\backend\arduino_bridge\liveguard_sensor_bridge.ino
3. Plug the Arduino into your laptop via USB cable.
4. Go to Tools -> Board -> Select 'Arduino Uno' (or Nano).
5. Go to Tools -> Port -> Select your COM port (e.g., COM3, COM4).
6. Click the Upload button (Arrow icon).
7. Optional Test: Open Tools -> Serial Plotter, set baud rate to 115200 to see live heartbeats!

# 5. Step 4: Run the LiveGuard Edge AI on Your Laptop

1. Open PowerShell or Command Prompt in C:\LiveGuard
2. Install required Python packages:
   pip install pyserial websockets torch scipy numpy scikit-learn

3. Make sure to CLOSE the Arduino Serial Monitor / Serial Plotter so the COM port is free.

4. Start the Edge AI engine with live sensors:
   python -m backend.run_edge --source ARDUINO --port COM3

5. Or test in Mock Mode without any wires connected:
   python -m backend.run_edge --source MOCK

# 6. Understanding Terminal Outputs

When the pipeline is running, it segments every heartbeat and displays:
• Beat Number & Real-time Heart Rate (BPM)
• AI Diagnosis: [NORMAL BEAT] (Conf: 98.2%) or [ABNORMAL/ARRHYTHMIA] (Prob: 0.88)
• Total Arrhythmia Alerts count
• WebSocket server status streaming at ws://0.0.0.0:8765 for the web dashboard.

# 7. Troubleshooting & Common Fixes

• 'PermissionError / Access Denied on COM port':
   Fix: The Arduino Serial Monitor is open in Arduino IDE. Close it so Python can access the port.

• 'Leads-off detected warning':
   Fix: One or more electrode pads came loose from your skin. Re-press the gel pads.

• 'Signal looks like a flatline':
   Fix: Ensure AD8232 is powered by 3.3V, GND is shared, and OUTPUT is connected to Analog Pin A0.